Supplementary Material

Designing public playgrounds for inclusion: A scoping review of grey literature guidelines for Universal Design

Alice Moore*, Bryan Boyle, & Helen Lynch

* Corresponding Author

Correspondence to:

Alice Moore, PhD Candidate, Department of Occupational Science and Occupational Therapy, University College Cork, Ireland

110341541@umail.ucc.ie

Supplementary Material

#### Findings: Theme 3

#### Location, layout, and accessibility

For location, layout, and accessibility, design recommendations primarily pertained to maximising accessibility and inclusion in the built environment and were categorised into nine main issues, presented below.

For location, nineteen documents provided recommendations (Supplementary Table 2). Recommendations included the need to: (a) orientate and position the play space to utilise the site’s unique features (e.g., views, existing vegetation); (b) connect the play space with transport networks (e.g., bus, train, light rail); (c) include appropriate arrival facilities (e.g., pedestrian and cycle routes, accessible parking); (d) connect street footpaths and the play space entry with safe road crossing points; and (e) include an accessible path (without stair-only access) from/to the street, carpark and supporting facilities.

For parking and other ways to arrive, twenty-one documents provided recommendations (Supplementary Table 2). For parking, these included the need to: (a) ensure car parks comply with country specific standards; (b) include convenient and designated accessible parking close to playground; and (c) provide an accessible path of travel from the car park to the playground. For other ways to arrive, recommendations included the ned to: (a) locate the playground so that it connects with transport networks (e.g., bus, train, light rail, path/cycle networks); (b) consider including a bus parking/drop-off zone for larger playgrounds; and (c) include appropriate arrival facilities (e.g., pedestrian and cycle routes, bicycle racks).

For entryway and orientation, seventeen documents provided recommendations (Supplementary Table 2). Recommendations included the need to: (a) consider a formal entry point that is accessible and linked by an orientation path to help people connect between all points of entry and exit; (b) incorporate a pause point to observe the layout, become familiar with the space, and decide where to go first; (c) incorporate signage or a map to assist with navigation and decision-making; and (d) add a "parking lot" immediately inside the entry way to park prams, buggies, wheelchairs, and other adaptive equipment.

For surfacing, twenty-three documents provided recommendations (Supplementary Table 2). Recommendations included the need to: (a) utilise surfacing that complies with country specific standards; (b) provide a variety of play surfacing to provide a contrast between activity, fall, and circulation zones; (c) consider the intensity of use, climate, ground profile and ground soil/ conditions when selecting surfacing; (d) incorporate inclusive play surfaces (e.g., wet pour, asphalt, concrete, poured-in-place).

For pathways, twenty-four documents provided recommendations (Supplementary Table 2). Recommendations included the need to provide pathways that are: (a) accessible (e.g., free from steep slopes); (b) free from barriers (e.g., kerbing); (c) level; (d) composed of a firm, predictable surface; and (e) wide enough so that two people using a wheelchair or mobility device to pass each other with ease.

For wayfinding, twelve documents provided recommendations (Supplementary Table 2). Recommendations included the need to: (a) develop a wayfinding system to help people easily navigate the play space (e.g., consistent multi-sensory signals and cues; a hierarchy of pathways); (b) provide accessibility features to assist with wayfinding (e.g., tactile map; colour contrast); and (c) provide a clear logical layout from entry to exit with play zones arranged adjacent to a main orientation path.

For orientation in and around the space, fifteen documents provided recommendations (Supplementary Table 2). Recommendations included the need to: (a) link the play space to surrounding elements (e.g., pathways); (b) provide a connection between the play space and the surrounding landscape to broaden the play experience; (c) ensure that there is a “flow of play” from one item to the next; (d) ensure that there is enough space between equipment for all users to navigate and manoeuvre; and (e) divide the larger play space into areas that allow for delineation of activities (e.g., separate vigorous play from quiet play zones).

For perimeter boundary, eighteen documents provided recommendations (Supplementary Table 2). Specifically, it was recommended to consider the purpose of the perimeter or boundary for the specific location before making decisions about what is required, i.e., complete / partial / no boundary. If it is decided that a perimeter boundary is required, recommendations included the need to: (a) consider what perimeter boundaries can add in terms of aesthetics and play value; (b) provide a fence or natural boundary (e.g., landform; planting); (c) avoid horizontal components, entrapment spaces, sharp components, cables or wire fencing; and (d) choose gates that are difficult to open by a child but can be operated by an adult using a wheelchair or mobility device.

Finally, for supervision and surveillance, sixteen documents provided recommendations (Supplementary Table 2). Specifically, to minimise the risk of children leaving the playground unsupervised, recommendations included the need to: (a) install a perimeter boundary (per above); and (b) allow for adult sight lines so that caregivers can safely supervise and facilitate children’s participation in play when needed. For adult sight lines, the following was recommended: (i) ensure clear line of sight from the entryway to the play space and parking lot; (ii) maximize the use of see-through equipment; and (iii) offer shaded, accessible seating and supervision areas throughout the play environment with a clear line of sight for seated or standing users.

#### Support features

Support features largely pertained to supportive infrastructure and amenities to maximise inclusion in the built environment within and surrounding the play space. For support features, design recommendations were categorised into twelve main issues, discussed below.

For seating, twenty-four documents provided recommendations (Supplementary Table 2). Recommendations included the need to: (a) provide seating at regular intervals on a universal route; (b) orient seating for the best vantage points for caregivers to supervise children during play; (c) set seating back far enough to allow for enough clearance space adjacent to the seat to park a pram, wheelchair, or mobility device without blocking the circulation space or path; (d) offer accessible seating options (e.g., some seating with armrests, and some without, to allow someone in a wheelchair to transfer to them); and (e) locate seating so the user is comfortable in all seasons (e.g., shade).

For tables/ picnic tables, sixteen documents provided recommendations (Supplementary Table 2).  Recommendations included the need to: (a) locate tables on a universal route; (b) orient tables for the best vantage points for caregivers to supervise children during play; (c) locate tables to promote multigenerational socialization; (d) provide tables that allow for multipurpose use (e.g., chessboard painted on tabletop); (e) include square, round, or hexagonal shaped tables to suit wheelchair users so everyone can sit together (enable a person to roll up to the table and put their legs underneath without getting hit by the tabletop; allow space for more than one wheelchair); and (f) locate tables so the user is comfortable in all seasons.

For spaces to gather and socialise, twenty-six documents provided recommendations (Supplementary Table 2). Specifically, inclusive play spaces were recognised to provide good opportunities for adults to relax and socialise with others. In particular, the use of seating, play equipment, play features (e.g., boulders) or other landscaping features were recommended to create gathering spaces where different age groups might choose to congregate, wait, sit, and/or supervise. Moreover, it was recommended to provide shade over gathering places to extend use of playgrounds in hot weather and provide shelter in cold or windy weather.

For shade and sheltered areas, twenty-one documents provided recommendations (Supplementary Table 2). Specifically, supporting diverse comfort characteristics was recommended via planning for a range of environmental conditions with a mix of sunlight, shade, and wind protection.  Moreover, ensuring that the play space has adequate shade over key play areas and seating to protect from strong sunlight and exposure to inclement weather was recommended via canopies, sails shades, trees, and solid shade structures (e.g., pergola).

For toilets/ changing facilities, twenty-one documents provided recommendations (Supplementary Table 2). Specifically, the provision of accessible toilets was regarded as crucial to the development of inclusive play space. Insofar as possible, it was recommended to include toilets with: (a) an accessible pathway and wheelchair access; (b) tactile signage and Braille; (c) doors that are easy to open and lock; (e) an adult-sized change bench in at least one unisex cubicle; (f) baby changing facilities; and (g) regular maintenance. If it is not possible to install toilets and changing facilities, it was recommended to locate the play space near accessible facilities.

For litter bins, seven documents provided recommendations (Supplementary Table 2). Recommendations included the need to: (a) locate bins on accessible routes; (b) not locate bins in clear spaces next to benches (those spaces may be needed for people using wheelchairs); (c) not locate bins within eating areas; (d) include bins that can be operated with one hand (e.g., open tops, push doors); and (e) ensure bins are properly and regularly disposed.

For drinking fountains, eleven documents provided recommendations (Supplementary Table 2). Recommendations included the need to: (a) located drinking fountains on firm, level surfacing; (b) include at least two different fountain heights; and (c) provide drinking fountains that are easy to operate (e.g., lever, sensor).

For lighting, eight documents provided recommendations (Supplementary Table 2). Recommendations included the need to: (a) incorporate streetscape or playspace specific lighting to enhance use and provide safety; and (b) ensure supporting facilities are adequately lit (e.g., car park, toilets, access paths, seating areas).

For storage facilities, five documents (Supplementary Table 2) recommended storage facilities to improve accessibility for families needing to bring medical equipment (e.g., oxygen cylinders, changing bags).

For service/ assistance animals, three documents provided recommendations (Supplementary Table 2). Recommendations included the need to: (a) include a designated area on-site provided for animal care, ensuring that it is on a universal route; (b) provide spaces where service animals can relieve themselves and provide for appropriate waste disposal; (c) include signs for owners to dispose of service animal waste; (d) provide a water trough/ basin/ drinking fountain for use by service animals; and (e) provide nearby safe and shady places where service animals can wait and have a clear view of their handlers when they are not assisting them.

For signage, twenty-two documents provided recommendations (Supplementary Table 2). Recommendations included the need to: (a) comply with following internationally specified shapes in signboards (i.e., information signboards should be rectangular; warning signboards triangular; and interdiction signboards circular); (b) include signage that is easy to read and understand (e.g., multiple methods of communication; consistent wording, font, and colouring); and (c) incorporate signage that is welcoming and child friendly (e.g., include local children in the design of signage).

Finally, for web page/ social media, eight documents provided recommendations (Supplementary Table 2). Recommendations included the need to: (a) ensure that the dedicated web page for the playground is universally accessible (e.g., easy to navigate, read, and understand; provide the information in multiple formats); and (b) utilise a web page and social media platform to prepare a family to visit (e.g., include directions, information, and printable flashcards).

Supplementary Tables

Supplementary Table 1: Research on inclusive playground provision

| Focus of study | Reference(s) |
| --- | --- |
| Explored user perspectives | Bhuyan 2021; Black and Ollerton 2021; Hill and Chawla 2019; Lynch et al. 2020; Olsen and Dieser 2012; Prellwitz and Skär 2007, 2016; Ripat and Becker 2012; Stanton-Chapman and Schmidt 2019, 2017a, 2016; Sterman et al. 2019; Wenger et al. 2021 |
| Investigated the perspectives of municipality representatives | Lynch et al. 2020, 2019; Prellwitz and Tamm 1999; Prellwitz, Tamm, and Lindqvist 2001; Sterman et al. 2019; Van Melik and Althuizen 2020 |
| Reviewed policy documents | Lynch, Moore, and Prellwitz 2018; Van Melik and Althuizen 2020 |
| Analyzed guidelines | Kim, Kim, and Maeng 2018; Lynch, Moore, and Prellwitz 2018; Lynch et al. 2019; Olsen 2015 |
| Studied research evidence | Alsarawi 2020; Brown et al. 2021; Burke 2013; Casey 2017; Fernelius and Christensen 2017; Moore and Lynch 2015; Moore, Lynch, and Boyle 2020; Prellwitz and Lynch 2018 |
| Developed evaluation tools to assess playground design | Dalprà 2020; Gustat, Anderson, and Slater 2020; Lynch et al. 2019; Parker and Al-Maiyah 2021 |
| Proposed frameworks for creating inclusive playgrounds | Olsen 2015 |
| Evaluated playground design | Lynch et al. 2019; Olsen and Dieser 2012; Perry et al. 2018; Siu, Wong, and Lam 2017; Talay, Akpinar, and Belkayali 2010 |
| Investigated the number of children who played, and time spent playing on a universally designed playground and compared it to children playing on an inclusive playground | Stanton-Chapman et al. 2020 |
| Designed inclusive playgrounds | Stanton-Chapman and Schmidt 2017b; Yuen 2018 |

Supplementary Table 2: Research on inclusive playground provision

|  | Author(s), Publication year and location | Title | Design recommendations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Location, layout, and accessibility |  |  |  |  |  |  |  |  | Support features |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Location | Parking and other ways to arrive | Entryway and orientation | Surfacing | Pathways | Wayfinding | Orientation in and around the space | Perimeter boundary | Supervision and surveillance | Seating | Tables/ Picnic tables | Spaces to gather and socialise | Shade and sheltered areas | Toilets/ Changing facilities | Litter bins | Drinking fountains | Lighting | Storage facilities | Service and assistance animals | Signage | Web page/ social media |
| 1 | Alison and Wheway 2004 UK | Can play, will play: Disabled children and access to outdoor playgrounds | ● | ● | ● | ● | ● | ● |  |  |  | ● |  | ● |  | ● |  |  |  |  |  |  |  |
| 2 | DESSA 2007 Ireland | Play for all: Providing play facilities for disabled children |  |  |  | ● | ● |  | ● |  | ● | ● |  | ● | ● |  |  |  |  |  |  | ● |  |
| 3 | Shackell et al. 2008 UK | Design for play: a guide to creating successful play spaces | ● |  |  | ● | ● |  |  | ● | ● | ● |  | ● | ● | ● |  |  |  |  |  |  |  |
| 4 | Touched by Olivia 2012 Australia | The principles for inclusive play |  |  |  |  |  |  |  |  |  | ● |  | ● | ● |  |  |  |  |  |  | ● | ● |
| 5 | Play Wales 2016 UK | Developing and managing play spaces: A community toolkit | ● | ● |  | ● | ● |  | ● | ● |  | ● |  | ● |  | ● |  |  | ● |  |  | ● |  |
| 6 | Yuen 2016 China | Inclusive play space guide: Championing better and more inclusive spaces in Hong Kong | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● |  | ● |  | ● |  | ● |  |
| 7 | Pooja Hiranandani, Krishnamoorthy, and Gopal 2016 India | Breaking barriers through play: Policy guidelines and a technical manual for making play spaces inclusive |  | ● | ● | ● | ● |  |  | ● |  | ● | ● | ● | ● | ● | ● | ● | ● | ● |  | ● |  |
| 8 | Play Wales and Alison John and Associates 2017 UK | Creating accessible play spaces: A toolkit | ● | ● | ● | ● | ● |  | ● | ● | ● | ● | ● | ● | ● | ● | ● |  | ● | ● |  | ● |  |
| 9 | Casey and Harbottle 2018 UK | Free to play: A guide to creating accessible and inclusive public play spaces | ● | ● | ● | ● | ● |  |  |  |  | ● | ● | ● | ● | ● |  |  |  | ● |  | ● | ● |
| 10 | Inclusive Play, KIDS, and Wagenfeld 2019 UK | PiPA: Plan inclusive Play Areas |  | ● | ● | ● | ● | ● |  | ● | ● | ● | ● | ● | ● | ● | ● | ● |  |  |  | ● | ● |
| 11 | Rick Hansen Foundation 2019 Canada | A guide to creating accessible play spaces | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● |  |  | ● | ● | ● |
| 12 | Christopher and Dana Reeve Foundation n.d. USA | Toolkit for building an inclusive community playground | ● | ● |  | ● | ● |  |  |  |  |  |  | ● | ● | ● |  |  |  |  |  |  |  |
| 13 | Rick Hansen Foundation n.d. Canada | Let's play toolkit: Creating inclusive play spaces for children of all abilities |  | ● | ● | ● | ● | ● | ● |  |  | ● | ● | ● | ● | ● | ● | ● |  |  |  | ● |  |
| 14 | Luce 2015 Australia | Inclusive design manual: A guide to creating play spaces which welcome children of all abilities |  |  |  |  | ● |  | ● |  | ● | ● |  | ● |  |  |  |  |  |  |  | ● |  |
| 15 | HAGS 2018 UK | Inclusive play design guide | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● |  |  | ● | ● | ● |
| 16 | Landscape Structures 2018 USA | Inclusive playspace design: Planning guide |  | ● |  | ● | ● |  |  | ● |  | ● |  | ● | ● | ● |  |  |  |  |  | ● |  |
| 17 | HAGS 2019 UK | Guide to creating inclusive playgrounds | ● | ● | ● |  |  |  |  | ● | ● |  | ● |  |  | ● |  |  | ● |  |  | ● | ● |
| 18 | Playworld 2019 USA | Inclusive play design guide | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● |  | ● |  |  | ● | ● | ● |
| 19 | Kompan Play Institute 2020 Denmark | Play for all: Universal design for inclusive playgrounds | ● | ● | ● | ● | ● |  | ● | ● |  | ● | ● | ● |  | ● |  |  |  |  |  | ● |  |
| 20 | Dunn, Moore, and Murray 2003 UK | Developing accessible play space: A good practice guide | ● | ● | ● | ● | ● | ● |  | ● | ● | ● | ● | ● | ● | ● |  |  | ● | ● |  | ● |  |
| 21 | State of Victoria 2007 Australia | The good play space guide: “I can play too” | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● |  | ● |  |  |  | ● |  |
| 22 | NSW Government 2019 Australia | Everyone can play: A guideline to create inclusive playspaces | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● |  |  | ● |  |
| 23 | Government of South Australia n.d. Australia | Inclusive play: Guidelines for accessible playspaces | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● | ● |  | ● | ● |  |  | ● | ● |
| 24 | PlayCore, GameTime, and Utah State University 2008 USA | EveryBODY plays! | ● | ● |  | ● | ● |  | ● | ● | ● | ● |  | ● | ● |  |  |  |  |  |  | ● |  |
| 25 | PlayCore and Utah State University 2016 USA | Me2: 7 principles of inclusive playground design |  | ● |  | ● | ● | ● | ● | ● | ● | ● |  | ● | ● | ● |  | ● |  |  |  | ● |  |
| 26 | CABE Space 2008 UK | Public space lessons: Designing and planning for play | ● |  |  |  |  |  |  |  |  |  |  | ● |  |  |  |  |  |  |  |  |  |
| 27 | Liedtke 2016 USA | Creating inclusive play and community spaces: An out-of-the-box approach to social and emotional inclusion | ● |  | ● | ● | ● |  |  |  |  | ● | ● | ● | ● |  |  |  | ● |  |  |  |  |

References

Alison, John, and Rob Wheway. 2004. Can Play Will Play: Disabled Children and Access to Outdoor Playgrounds. London: National Playing Fields Association. http://www.childrensplayadvisoryservice.org.uk/pdf_files/Publications/can_play_will_play-CPASwebsite.pdf

Alsarawi, Aeshah. 2020. "Inclusive Playgrounds: Concerted Efforts for Children with Disabilities in Saudi Arabia."  International Journal of Play 9 (4): 382-99. doi: 10.1080/21594937.2020.1843802.

Bhuyan, Md Rashed. 2021. "Geographies of Outdoor Play in Dhaka: An Explorative Study on Children’s Location Preference, Usage Pattern, and Accessibility Range of Play Spaces."  Children's Geographies. Advance online publication. doi: 10.1080/14733285.2021.1914823.

Black, Rosemary, and Janice Ollerton. 2021. "Playspace Users’ Experience of a Socially Inclusive Playspace: The case study of Livvi’s Place, Port Macquarie, New South Wales, Australia."  World Leisure Journal. Advance online publication. doi: 10.1080/16078055.2021.1894230.

Brown, Denver, Timothy Ross, Jennifer Leo, Ron Buliung, Celina H Shirazipour, Amy E Latimer, and Kelly P Arbour-Nicitopoulos. 2021. "A Scoping Review of Evidence-Informed Recommendations for Designing Inclusive Playgrounds."  Frontiers in Rehabilitation Sciences 2. doi: 10.3389/fresc.2021.664595.

Burke, Jenene. 2013. "Just for the Fun of It: Making Playgrounds Accessible to All Children." World Leisure Journal 55 (1): 83-95. doi:10.1080/04419057.2012.759144.

Casey, Teresa, and Harry Harbottle. 2018. Free to Play: A Guide to Creating Accessible and Inclusive Public Play Spaces. Scotland: Inspiring Scotland, Play Scotland and the Nancy Ovens Trust. https://www.inspiringscotland.org.uk/publication/free-play-guide-creating-accessible-inclusive-public-play-spaces/

CABE Space. 2008. "Public Space Lessons: Designing and Planning for Play." Accessed 16 June 2020. https://www.designcouncil.org.uk/sites/default/files/asset/document/designing-and-planning-for-play.pdf

Casey, Theresa. 2017. "Outdoor Play and Learning in the Landscape of Children’s Rights." In The SAGE Handbook of Outdoor Play and Learning, edited by Tim Waller, Eva Arlemalm-Hagser, Ellen Beate Hansen Sandseter, Libby Lee-Hammond, Kristi Lekies and Shirley Wyver, 362-377. London: SAGE Publications.

Christopher and Dana Reeve Foundation. n.d. "Toolkit for Building an Inclusive Community Playground." Accessed 22 June 2020. http://s3.amazonaws.com/reeve-assets-production/Quality-of-Life-Toolkit-2-1.pdf

Dalprà, Michela. 2020. "Inclusive Playgrounds: A Reality or a Utopia in Our Cities?"  Journal of Civil Engineering and Architecture 14: 635-43. doi: 10.17265/1934-7359/2020.12.001.

DESSA. 2007. "Play for All: Providing Play Facilities for Disabled Children." Accessed 16 June 2020. https://www.dessa.ie/wp-content/uploads/2021/03/Play-for-All.pdf

Dunn, Karen, Michele Moore, and Pippa Murray. 2003. Developing Accessible Play Space: A Good Practice Guide. London: Office of the Deputy Prime Minister. http://www.carltd.com/sites/carwebsite/files/Accessible%20Play%20Space.pdf

Fernelius, Courtney L., and Keith M. Christensen. 2017. "Systematic Review of Evidence-Based Practices for Inclusive Playground Design." Children, Youth and Environments 27 (3): 78-102. doi:10.7721/chilyoutenvi.27.3.0078.

Government of South Australia. n.d. "Inclusive Play: Guidelines for Accessible Playspaces." Accessed 22 June 2020.  https://dhs.sa.gov.au/__data/assets/pdf_file/0018/85023/Inclusive-Play-GuidelinesV2.pdf

Gustat, Jeanette, Christopher E Anderson, and Sandy J Slater. 2020. "Development and Testing of a Brief Play Space Audit Tool."  Journal of Physical Activity and Health 17 (1): 13-20. doi: 10.1123/jpah.2018-0686.

HAGS. 2018. "Inclusive Play: Design Guide." Accessed 29 June 2020. https://issuu.com/hags-uk/docs/hags_-_inclusive_play_-_design_guid

HAGS. 2019. "Inclusive Play: Guide to Creating Inclusive Playgrounds." Accessed 29 June 2020. https://www.hags.com/en-us/designing-inclusive-playgrounds/download-links-inclusive-playground-pack

Hill, Catherine, and Louise Chawla. 2019. ""Fun, Novel, Safe, and Inclusive": Children's Design Recommendations for Playgrounds for All Abilities."  Children, Youth & Environments 29 (1): 105-15. doi: 10.7721/chilyoutenvi.29.1.0105.

Inclusive Play, KIDS, and Amy Wagenfeld. 2019. "PiPA: Plan Inclusive Play Areas." Accessed 16 June 2020. https://www.inclusiveplay.com/wp-content/uploads/2021/02/PiPA-Interactive-Form.pdf

Kim, Yun-Geum, Hana Kim, and Soo-hyun Maeng. 2018. "Characteristics of Inclusive Playground Guidelines." Journal of the Korean Institute of Landscape Architecture 46 (6): 75-84. doi: 10.9715/KILA.2018.46.6.075.

Kompan Play Institute. 2020. "Play for All: Universal Design for Inclusive Playgrounds." Accessed 25 June 2020. https://www.kompan.ie/play-for-all-universal-designs-for-inclusive-playgrounds

Landscape Structures. 2018. "Inclusive Playspace Design: Planning Guide." Accessed 16 June 2020. http://viewer.zmags.com/publication/c878a7ae#/c878a7ae/1

Liedtke, Helena. 2016. "Creating Inclusive Play and Community Spaces: An Out-of-the-Box Approach to Social and Emotional Inclusion." Accessed 16 June 2020.  https://www.communityinclusion.org/inclusiveplayspaces/inclusive_playspaces.pdf

Luce, Brian  2015. "Inclusive Design Manual: A Guide to Creating Play Spaces Which Welcome Children of All Abilities." Accessed 16 June 2020. https://playgroundideas.org/dashboard/

Lynch, Helen, Alice Moore, Claire Edwards, and Linda Horgan. 2019. Community Parks and Playgrounds: Intergenerational Participation Through Universal Design. Dublin: National Disability Authority. http://nda.ie/Publications/Others/Research-Promotion-Scheme/Community-Parks-and-Playgrounds-Intergenerational-Participation-through-Universal-Design1.pdf

Lynch, Helen, Alice Moore, Claire Edwards, and Linda Horgan. 2020. "Advancing Play Participation for All: The Challenge of Addressing Play Diversity and Inclusion in Community Parks and Playgrounds." British Journal of Occupational Therapy 83 (2): 107-117. doi:10.1177/0308022619881936.

Lynch, Helen, Alice Moore, and Maria Prellwitz. 2018. "From Policy to Play Provision: Universal Design and the Challenges of Inclusive Play." Children, Youth and Environments 28 (2): 12-34. doi: 10.7721/chilyoutenvi.28.2.0012. doi:10.7721/chilyoutenvi.28.2.0012.

Moore, Alice, and Helen Lynch. 2015. "Accessibility and Usability of Playground Environments for Children Under 12: A Scoping Review." Scandinavian Journal of Occupational Therapy 22 (5): 331-344. doi:10.3109/11038128.2015.1049549.

Moore, Alice, Helen Lynch, and Bryan Boyle. 2020. "Can Universal Design Support Outdoor Play, Social Participation, and Inclusion in Public Playgrounds? A Scoping Review."  Disability and Rehabilitation, 1 – 22. Advance online publication. doi: 10.1080/09638288.2020.1858353.

NSW Government. 2019. "Everyone Can Play: A Guideline to Create Inclusive Playspaces." Accessed 16 June 2020. https://everyonecanplay.nsw.gov.au/sites/default/files/2019-02/Everyone%20Can%20Play%20Guideline.pdf

Olsen, Heather M. 2015. "Planning Playgrounds: A Framework to Create Safe and Inclusive Playgrounds."  Journal of Facility Planning, Design & Management 3 (1): 57-71.

Olsen, Heather M., and Rodney B. Dieser. 2012. "“I Am Hoping You Can Point Me in the Right Direction Regarding Playground Accessibility”: A Case Study of a Community Which Lacked Social Policy Toward Playground Accessibility." World Leisure Journal 54 (3): 269-279. doi:10.1080/04419057.2012.702456.

Parker, Ruth, and Sura Al-Maiyah. 2022. "Developing an Integrated Approach to the Evaluation of Outdoor Play Settings: Rethinking the Position of Play Value."  Children's Geographies 20 (1): 1 – 23. doi: 10.1080/14733285.2021.1912294.

Perry, Meredith A, Hemakumar Devan, Harry Fitzgerald, Karen Han, Li-Ting Liu, and Jack Rouse. 2018. "Accessibility and usability of parks and playgrounds." Disability and Health Journal 11 (2): 221-9. doi:10.1016/j.dhjo.2017.08.011.

Play Wales. 2016. "Developing and Managing Play Spaces: A Community Toolkit." 3rd ed. Accessed 16 June 2020. http://www.playwales.org.uk/login/uploaded/documents/Publications/Community%20Toolkit%202016.pdf

Play Wales, and Alison John and Associates. 2017. "Creating Accessible Play Spaces: A Toolkit." Accessed 16 June 2020. https://www.playwales.org.uk/login/uploaded/documents/Publications/Creating%20accessible%20play%20spaces.pdf

PlayCore, and Utah State University. 2016. Me2: 7 Principles of Inclusive Playground Design. Chatanooga, TN: PlayCore. https://www.playcore.com/programs/me2

PlayCore, GameTime, and Utah State University. 2008. EveryBODY Plays!. Chatanooga, TN: PlayCore.  http://www.google.ie/url?sa=t&rct=j&q=&esrc=s&source=web&cd=54&ved=0ahUKEwj60KTg2avKAhXD-A4KHTRKDlw4MhAWCC4wAw&url=http%3A%2F%2Focalafl.iqm2.com%2FCitizens%2FFileOpen.aspx%3FType%3D4%26ID%3D1491&usg=AFQjCNE6aR2p_WwwzEP-EnEFrFLJ4MUnKA

Playworld. 2019. "Inclusive Play Design Guide." Accessed 16 June 2020. https://odellengineering.com/wp-content/uploads/2019/04/Playworld-Inclusive-Design-Guide.pdf

Pooja Hiranandani, Vinita S, Kavitha Krishnamoorthy, and Ritu Gopal. 2016. Breaking Barriers Through Play: Policy Guidelines and a Technical Manual for Making Play Spaces Inclusive. Chennai: Kilikili. https://issuu.com/kilikili-inclusiveplayspace/docs/kilikili_technical_manual_28nov16_w kilikili

Prellwitz, Maria, and Helen Lynch. 2018. "Universal design for social inclusion: playgrounds for all." In Seen and Heard: Exploring Participation, Engagement and Voice for Children with Disabilities, edited by Miriam Twomey and Clare Carroll, 267-296. Oxford: Peter Lang.

Prellwitz, Maria, and Lisa Skär. 2007. "Usability of Playgrounds for Children with Different Abilities." Occupational Therapy International 14 (3): 144-155. doi:10.1002/oti.230.

Prellwitz, Maria, and Lisa Skär. 2016. "Are Playgrounds a Case of Occupational Injustice? Experiences of Parents of Children with Disabilities." Children, Youth and Environments 26 (2): 28-42. doi:10.7721/chilyoutenvi.26.2.0028.

Prellwitz, Maria, and Maare Tamm. 1999. "Attitudes of Key Persons to Accessibility Problems in Playgrounds for Children with Restricted Mobility: A study in a Medium-Sized Municipality in Northern Sweden." Scandinavian Journal of Occupational Therapy 6 (4): 166-173. doi:10.1080/110381299443645.

Prellwitz, Maria, Maare Tamm, and Rafael Lindqvist. 2001. "Are Playgrounds in Norrland (Northern Sweden) Accessible to Children with Restricted Mobility?" Scandinavian Journal of Disability Research 3 (1): 56-68. doi:10.1080/15017410109510768.

Rick Hansen Foundation. 2019. "A Guide to Creating Accessible Play Spaces." Accessed 16 June 2020. https://www.rickhansen.com/sites/default/files/downloads/sch-35913-guide-creating-accessible-play-spaceswebaccessible.pdf

Rick Hansen Foundation. n.d. "Let's Play Toolkit: Creating Inclusive Play Spaces for Children of All Abilities." Accessed 26 June 2020. https://www.rickhansen.com/sites/default/files/downloads/letsplaytoolkit.pdf

Ripat, Jacquie, and Pam Becker. 2012. "Playground Usability: What Do Playground Users Say?" Occupational Therapy International 19 (3): 144-153. doi:10.1002/oti.1331.

Shackell, Aileen, Nicola Butler, Phil Doyle, and David Ball. 2008. Design for Play: A Guide to Creating Successful Play Spaces. Nottingham: The Department for Children, Schools and Families  and the Department for Culture, Media and Sport. https://www.playengland.org.uk/resources/design-for-play.aspx

Siu, Kin Wai Michael, Yi Lin Wong, and Mei Seung Lam. 2017. "Inclusive Play in Urban Cities: A Pilot Study of the Inclusive Playgrounds in Hong Kong."  Procedia engineering 198: 169-75. doi: 10.1016/j.proeng.2017.07.080

Stanton-Chapman, Tina L., and Eric L. Schmidt. 2016. "Special Education Professionals' Perceptions Toward Accessible Playgrounds." Research and Practice for Persons with Severe Disabilities 41 (2): 90-100. doi:10.1177/1540796916638499.

Stanton-Chapman, Tina L., and Eric L. Schmidt. 2017a. "Caregiver Perceptions of Inclusive Playgrounds Targeting Toddlers and Preschoolers with Disabilities: Has Recent International and National Policy Improved Overall Satisfaction?" Journal of Research in Special Educational Needs 17 (4): 237-246. doi:10.1111/1471-3802.12381.

Stanton-Chapman, Tina L., and Eric L. Schmidt. 2017b. "Creating an Inclusive Playground for Children of All Abilities: West Fork Playground in Cincinnati, Ohio."  Children, Youth & Environments 27 (3): 124-37. doi: 10.7721/chilyoutenvi.27.3.0124.

Stanton-Chapman, Tina L., and Eric L. Schmidt. 2019. "Building Playgrounds for Children of "All" Abilities: Legal Requirements and Professional Recommendations."  Early Childhood Education Journal 47 (5): 509-17. doi: 10.1007/s10643-019-00947-3.

Stanton-Chapman, Tina L, Eric L Schmidt, Carla Rhoades, and Lindsey Monnin. 2020. "An Observational Study of Children Playing on an Inclusive Playground and on an Universal Playground." International Journal of Social Policy and Education 2 (2): 37-47.

State of Victoria. 2007. "The Good Play Space Guide: I Can Play Too." Accessed 16 June 2020. https://sport.vic.gov.au/resources/documents/good-play-space-guide-i-can-play-too

Sterman, Julia J., Geraldine A. Naughton, Anita C. Bundy, Elspeth Froude, and Michelle A. Villeneuve. 2019. "Planning For Outdoor Play: Government and Family Decision-Making." Scandinavian Journal of Occupational Therapy 26 (7): 484-495. doi:10.1080/11038128.2018.1447010.

Talay, Likden, Nevin Akpinar, and Nur Belkayali. 2010. "Barriers to Playground Use for Children with Disabilities: A Case from Ankara, Turkey." African Journal of Agricultural Research 5 (9): 848-855. doi:10.5897/ajar09.779.

Touched by Olivia. 2012. "The Principles for Inclusive Play." Accessed 16 June 2020. http://www.touchedbyolivia.com.au/wp-content/uploads/The-6-Principles-for-Inclusive-Play.pdf

Van Melik, Rianne, and Nick Althuizen. 2020. "Inclusive Play Policies: Disabled Children And Their Access To Dutch Playgrounds."  Tijdschrift voor economische en sociale geografie. doi: 10.1111/tesg.12457

Wenger, Ines, Christina Schulze, Ulrica Lundström, and Maria Prellwitz. 2021. "Children’s perceptions of playing on inclusive playgrounds: A qualitative study."  Scandinavian Journal of Occupational Therapy 28 (2): 136-46. doi:10.1080/11038128.2020.1810768.

Yuen, Chris H. C.. 2016. Inclusive Play Space Guide: Championing Better and More Inclusive Spaces in Hong Kong. Hong Kong: Playright Children’s Play Association. https://playright.org.hk/wp-content/uploads/2018/12/Playright-Inclusive-Play-Space-Guide.pdf

Yuen, Chris H. C. 2018. "Championing and Implementing the First Inclusive Play Space in Hong Kong."  Children, Youth & Environments 28 (2): 35-45. doi: 10.7721/chilyoutenvi.28.2.0035.