# Biological Hydrogen Methanation Systems – An overview of design and efficiency.

Authors

Davis Rusmanis,, Richard O’Sheaa,b,, David M. Walla,b, Jerry D. Murphya,b

# Supplementary data

The purpose of Box S1 is to provide a comparison for use of methane (CH4) and hydrogen (H2) as an energy carrier gas. An energy transmission of 1GWth was chosen as an arbitrary value of energy required to be transmitted

BOX S1

|  |  | Methane | Hydrogen |
| --- | --- | --- | --- |
| Density | kg/m3 | 6.585 | 0.8085 |
| Viscosity | CP | 1100 | 880 |
| LHV | MJ/kg | 50 | 119.9 |

Gas grid pressure: 1MPa

Temperature: 25°C

Energy transmission: 1,000MWth (Arbitrary number chosen for comparison.)

Hydrogen

Mass Flow:		
1000119.9 =8.34kgs

Volume Flow:		
8.340.8085=10.315m3s

Pressure Drop:		
=4.0156kPakmpipe

Specific work:		
4.01560.8085= 4.967kJkgH2 . kmpipe

Power:			
4.967*8.34=41.42kWkmpipe

Methane

Mass Flow:		
100050=20kgs

Volume Flow:		
206.585=3.037m3s

Pressure Drop:		
1.477kPakmpipe

Specific work:		
1.4776.858=0.224 kJkgCH4 . kmpipe

Power: 			
0.224*20=4.48kW kmpipe

MaREI Centre, Environmental Research Institute (ERI), University College Cork (UCC), Ireland.

School of Engineering, UCC, Ireland.

Corresponding author at: School of Engineering, University College Cork, Cork, Ireland. E-mail address: richard.oshea@ucc.ie (Richard O’Shea)