Supplementary Material

Chandran et al., Off-axis cavity enhanced absorption spectroscopy of 14NH3 in air using a gain-switched frequency comb at 1.514 µm, Sensors MDPI.

Table S1. Absorption strengths and their relative uncertainties of NH3 lines between 6604 and 6607 cm−1 and a list of some common gases that also exhibit absorption features in this wavenumber region. The data in columns 2–5 was taken from Ref. [26].

| Molecule | No. of Lines between 6604 and 6607 cm−1 | Max Absorption Strength, Smax, in that Region [cm/molecule] | Relative Uncertainty % | Transition Wavenumber [cm−1] |
| --- | --- | --- | --- | --- |
| NH3 | 68 | 2.38 10−21 3.31 10-21 1.45 10−21 3.61 10−21 3.15 10−21 | 5 < S < 10 5 < S < 10 2 < S < 5 5 < S < 10 5 < S < 10 | 6604.728 6605.104 6605.190 6605.609 6605.652 |
| CO2 | 50 | 5.16 10−28 |  | 6606.8503 |
| OCS | 57 | 7.01 10−26 |  | 6606.4128 |
| H2O | 33 | 1.06 10−25 |  | 6605.9890 |
| H2S | 7 | 1.46 10−25 |  | 6606.4042 |
| CH4 | 76 | 1.53 10−25 3.61 10−25 |  | 6604.0564 6604.0770 |
| C2H2 | 14 | 2.29 10−21 |  | 6605.1401 |

Figure S1. Measurements of NH3 absorption spectra for three different mixing ratios in 1003 mbar of air. The measurement conditions were the same as for the spectrum in Figure 4 shown in the main text. Insert: Linearly increasing absorption coefficient in the maxima of the two strongest bands in this region. The graph illustrates the reproducibility and linearity of the experimental approach.

Figure S2. HITRAN simulation of an absorption spectrum (resolution of 0.15 cm−1) of NH3 at a mixing ratio of 
3.7 ppmv and H2O at a relative humidity (RH) of 90% in air at 1 atm. The feature at 6605.6 cm−1 and the background under the two main water bands are due to NH3 absorption at the mixing ratio stated. Through a fit of Equation (2) to the spectrum the outlined approach is still selective to NH3.