# What physicochemical properties of biochar facilitate interspecies electron transfer in anaerobic digestion: a case study of digestion of whiskey by-products

Chen Deng 1, 2, Richen Lin 1, 2, , Xihui Kang 1, 2, Benteng Wu 1, 2, David M Wall 1, 2, Jerry D Murphy 1, 2

1 MaREI Centre, Environmental Research Institute, University College Cork, Cork, Ireland

2 Civil, Structural and Environmental Engineering, School of Engineering and Architecture, University College Cork, Cork, Ireland

The supporting information contains 2 tables as referred to in the main manuscript:

Table S1 Calculation of mass and energy balance for anaerobic digestion (AD) of draff.

Table S2 Calculation of mass and energy balance for pyrolysis of draff using data adapted from literature [1].

Table S1 Calculation of mass and energy balance for anaerobic digestion (AD) of draff.

| Parameters | Acronyms | Units | Values | Equations | References |
| --- | --- | --- | --- | --- | --- |
| Mass flow of draff (wwt) | m draff | kg | 1.000 |  |  |
| VS in draff | VS draff | kg | 0.307 | The VS content in draff is measured as 30.7%. | This study |
| TS in draff | TS draff | kg | 0.317 | The TS content in draff is measured as 31.7%. | This study |
| CH4 production (accounting for 78% of the biogas) | Y CH4 | L | 92.806 | Y CH4 = VS draff × 302.3 L/kg VS | This study |
| Mass flow of CH4 | m CH4 | kg | 0.066 | m CH4 = YCH4 × 0.716 kg/m3 ÷ 1000 |  |
| CO2 production (accounting for 22% of the biogas) | Y CO2 | L | 61.871 | YCO2 = YCH4 ÷ 60% × 40% |  |
| Mass flow of CO2 | m CO2 | kg | 0.123 | m CO2 = Y CO2 × 1.980 kg/m3 ÷ 1000 |  |
| Mass flow of digestate | m digestate | kg | 0.811 | m digestate = m draff – m CH4 – m CO2 |  |
| Energy value of CH4 | Q CH4 | MJ | 3.69 | Q CH4 = Y CH4 ÷ 1000 × 39.8 MJ/m3, in which 39.8 MJ/m3 is the HHV of CH4 at standard conditions of 273 K and 1 bar. |  |
| Electricity consumption for biogas production | E IN AD | MJ | 0.06 | E IN AD = Y CH4 ÷ 1000 × (0.18 kWh/m3 × 3.6) | [2] |
| Heat consumption for biogas production | H IN AD | MJ | 0.05 | H IN AD = Y CH4 ÷ 1000 × (0.14 kWh/m3 × 3.6) | [2] |
| Electricity consumption for digestate segregation | E IN digestate segregation | MJ | 0.01 | E IN digestate segregation = m digestate ÷ 1000 × (3.5 kWh/t × 3.6) | [3] |
| Total primary energy consumption in the AD system | Q IN P AD | MJ | 0.22 | Q IN P AD = (E IN AD + E IN digestate segregation) × 2.3 + H IN AD × 1.1, in which 2.3 and 1.1 are primary energy conversion factors for electricity and heat, respectively. | [4] |
| Energy output of the system | Q OUT AD | MJ | 3.69 | Q OUT AD = Q CH4 |  |
| Net energy gain | Q NG AD | MJ | 3.47 | Q NG AD = Q OUT AD – Q IN P AD |  |

Table S2 Calculation of mass and energy balance for pyrolysis of draff using data adapted from literature [1].

| Parameters | Acronyms | Units | Values | Equations |
| --- | --- | --- | --- | --- |
| Mass flow of draff (wet) | m draff | kg | 1.450 |  |
| TS in draff | TS draff | kg | 0.460 | TS draff = m draff × 31.7% |
| Biochar production | m char | kg | 0.078 | m char = TS draff × 17% |
| Bio-oil production | m oil | kg | 0.244 | m oil = TS draff × 53% |
| Syngas production | m syngas | kg | 0.138 | m syngas = TS draff × 30% |
| Energy value of biochar | Q char | MJ | 1.95 | Q char = m char × 25.0 MJ/kg, in which 25.0 MJ/kg is the HHV of biochar. |
| Energy value of bio-oil | Q oil | MJ | 4.72 | Q oil = m oil × (1-15%) × 22.8 MJ/kg, in which 15% is the moisture content in bio-oil and 22.8 MJ/kg is the HHV of moisture free bio-oil. |
| Energy value of syngas | Q syngas | MJ | 0.19 | Q syngas = m syngas × 1.4 MJ/kg, in which 1.4 MJ/kg is the HHV of syngas. |
| Energy consumption for grinding | E IN grinding | MJ | 0.10 | E IN grinding = m draff × 0.07 MJ/kg |
| Energy consumption for drying | E IN drying | MJ | 2.77 | E IN FR drying = (m draff – TS draff) × 2.8 MJ/kg H2O |
| Energy consumption for pyrolysis | E IN PY | MJ | 0.61 | E IN PY = m oil × 2.5 MJ/kg bio-oil |
| Energy to produce the ice required for bio-oil condensation | E IN ice | MJ | 0.52 | E IN ice = m draff × 0.36 MJ/kg |
| Total energy consumption of the pyrolysis process | Q IN PY | MJ | 4.00 | Q IN PY = E IN grinding + E IN drying + E IN PY + E IN ice |
| Energy output of the pyrolysis system | Q OUT PY | MJ | 4.91 | Q OUT PY = Q oil + Q syngas |
| Net energy gain | Q NG PY | MJ | 0.91 | Q NG PY = Q OUT PY – Q IN P PY |

Note: The data is adapted from the pyrolysis of wheat and barley spent grain operated at 520 °C.

References

[1] A. Sanna, S. Li, R. Linforth, K.A. Smart, J.M. Andresen. Bio-oil and bio-char from low temperature pyrolysis of spent grains using activated alumina. Bioresour Technol. 102 (2011) 10695-703.

[2] T.T.Q. Vo, D.M. Wall, D. Ring, K. Rajendran, J.D. Murphy. Techno-economic analysis of biogas upgrading via amine scrubber, carbon capture and ex-situ methanation. Appl Energy. 212 (2018) 1191-202.

[3] E. Tampio, S. Marttinen, J. Rintala. Liquid fertilizer products from anaerobic digestion of food waste: mass, nutrient and energy balance of four digestate liquid treatment systems. Journal of Cleaner Production. 125 (2016) 22-32.

[4] A. Esser, F. Sensfuss. Review of the default primary energy factor (PEF) reflecting the estimated average EU generation efficiency referred to in Annex IV of Directive 2012/27/EU and possible extension of the approach to other energy carriers. Fraunhofer-Institut für System-und Innovationsforschung (ISI), Karlsruhe, Germany, 2016.

Corresponding author: Dr. Richen Lin, MaREI Centre, Environmental Research Institute, University College Cork, Ireland. Tel.: +353 (0)21 490 1948. Email: richen.lin@ucc.ie