ANNEX

Table 1: Summary of assumptions used for biogas plant and biomethanation reactor

|  | Unit | Grass | Pig Slurry | SHW | OFMSW | Seaweed |
| --- | --- | --- | --- | --- | --- | --- |
| CH4:CO2 ratio in biogas |  | 55%:45% | 65%:35% | 55%:45% | 60%:40% | 55%:45% |
| m3 biomethane yield/ton feedstock |  | 59.4 | 14.4 | 41 | 66 | 24.6 |
| capital cost | €/t/a | 162 | 102 | 154 | 325 | 164 |
| Ton feedstock/year |  | 19000 | 77000 | 27000 | 19000 | 46000 |
| Storage pit | €/t/a | 30 |  |  |  | 30 |
| Technology used |  | CSTR | CSTR | CSTR | Batch process | CSTR |
| Maintenance and overhead | €/t | 5 | 5 | 10 | 25 | 15 |
| Digestate disposal | €/t |  |  |  | 4 |  |
| Electrical demand of biogas plant | kWeh/t | 10 | 10 | 10 | 6 | 10 |
| electricity price | €/kWeh | 0.15 | 0.15 | 0.15 | 0.15 | 0.15 |
| Cost of feedstock | €/t | 17 |  |  |  | 50 |
| Gate fee | €/t |  |  | 20 | 70 |  |
| Cost of capital | % | 6% |  |  |  |  |
| Life time | (years) | 15 |  |  |  |  |
| electricity demand for biomethanation | kWh/m3 | 1 |  |  |  | 1 |
| Gas grid connection | € | €300,000 |  |  |  |  |
| CNG service station | € | €500,000 |  |  |  |  |

Table 2: Capital investment (€) of renewable gas

|  | Grass | Pig Slurry | SHW | OFMSW | Seaweed |
| --- | --- | --- | --- | --- | --- |
| Biogas plant | 3078000 | 7854000 | 4158000 | 6175000 | 7544000 |
| storage | 570000 |  |  |  | 1380000 |
| Methanation reactor | 3000000 | 3000000 | 3000000 | 3000000 | 3000000 |
| Gas Grid connection | 30000 | 300000 | 300000 | 300000 | 300000 |
| CNG service station | 500000 | 500000 | 500000 | 500000 | 500000 |
| Total capital cost | 7178000 | 11654000 | 7958000 | 9975000 | 12724000 |

| Table 3: Annual cost (€) of biogas production |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Grass | Pig Slurry | SHW | OFMSW | Seaweed |
| Maintenance and overhead | 95000 | 385000 | 270000 | 475000 | 690000 |
| Electric demand of biogas plant | 28500 | 115500 | 40500 | 17100 | 69000 |
| Operation and maintenance cost of methanation | 90000 | 90000 | 90000 | 90000 | 90000 |
| Plant operation |  |  |  |  |  |
| Substrate cost | 323000 | 0 | 0 | 0 | 280000 |
| Digestate disposal | 0 | 0 | 0 | 76000 |  |
| Cost of capital | 739067 | 1199928 | 819378 | 1027054 | 1310098 |
| Depreciation fund | 202600 | 361800 | 238600 | 305833 | 351467 |
| Total annual cost | 1478167 | 2152228 | 1458478 | 1990987 | 2790565 |
| Income from gate fee | 0 | 0 | 540000 | 1330000 | 0 |
| Annual cost of renewable gas production | 1478167 | 2152228 | 918478 | 660987 | 2790565 |