# Supplementary Material

Table S1 Mooring system specifications [23]

| Mooring line properties |  | Value |
| --- | --- | --- |
| Number of mooring lines | - | 3 |
| Line material and type | - | R3 - Studless Chain |
| Length of a mooring line (unstretched) | m | 850 |
| Nominal diameter of chain | m | 0.185 |
| Dry mass density | kg/m | 685 |
| Axial stiffness | MN | 3,270 |
| Depth to fairleads / anchors below still water level | m | 14 / 200 |
| Radius to fairleads / anchors from center of platform | m | 58 / 837.6 |
| Minimum breaking strength | kN | 22,286 |

Table S2. Acceleration of nacelle, hydrogen storage tanks and electrolyzer under difference design load cases

| DLC/Acceleration |  | Nacelle Ax | Nacelle Ay | Nacelle Az | Tank Ax | Tank Ay | Tank Az | PEM Ax | PEM Ay | PEM Az |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.6_12mps | Max (m/s2) | 1.487 | 0.164 | 0.775 | 1.120 | 0.021 | 0.894 | 0.953 | 0.025 | 0.802 |
|  | Min (m/s2) | -1.496 | -0.151 | -0.859 | -1.09 | -0.01 | -0.875 | -0.891 | -0.028 | -0.783 |
|  | Std (m/s2) | 0.389 | 0.038 | 0.243 | 0.303 | 0.004 | 0.263 | 0.251 | 0.006 | 0.243 |
| 1.6_24mps | Max (m/s2) | 1.632 | 0.451 | 0.976 | 1.334 | 0.030 | 0.981 | 1.118 | 0.080 | 0.928 |
|  | Min (m/s2) | -1.902 | -0.490 | -1.048 | -1.178 | -0.038 | -0.919 | -1.028 | -0.071 | -0.927 |
|  | Std (m/s2) | 0.424 | 0.114 | 0.276 | 0.336 | 0.008 | 0.269 | 0.285 | 0.019 | 0.262 |
| 6.1_yaw 8° | Max (m/s2) | 2.125 | 1.531 | 1.047 | 1.460 | 0.093 | 0.982 | 1.229 | 0.328 | 0.985 |
|  | Min ((m/s2) | -1.755 | -1.622 | -1.150 | -1.270 | -0.090 | -0.944 | -1.152 | -0.387 | -0.982 |
|  | Std (m/s2) | 0.496 | 0.376 | 0.298 | 0.364 | 0.021 | 0.284 | 0.317 | 0.080 | 0.281 |