Supplementary file

Assessment of integration of methane-reduced ceria chemical looping CO2/H2O splitting cycle to an oxy-fired power plant

Azharuddin Farooqui1, 2, 3, Archishman Bose4, Marta Boaro5, Jordi Llorca3, and Massimo Santarelli1

1Department of Energy, Politecnico di Torino, Italy

2German Aerospace Center (DLR), Institute of Engineering Thermodynamics, 70569,

Stuttgart, Germany.

3Institute of Energy Technologies, Department of Chemical Engineering and Barcelona Research Center in Multiscale Science and Engineering, Universitat Politècnica de Catalunya, Barcelona, Spain

4Marine and Renewable Energy Research (MaREI) Centre, Environmental Research Institute, School of Engineering and Food Science, University College Cork, Lee Road, Cork, T23 XE10, Ireland

5Polytechnic Department of Engineering and Architecture (DPIA), Universita di Udine, Italy

Corresponding email id: azharuddin.xxx@polito.it; azharuddin0613@gmail.com

Table S1 Rate and integral expressions for different solid-state kinetic models [43] based on X, which indicates the conversion, f(X) represents the differential form of the kinetic model, and g(X) is the integral form of the model.

Figure S1. XRD images of fresh and used samples of ceria with methane reduction at different temperatures and concentration.

Figure S2. (a) and (b) represents microstruture of 900°C and 30%CH4; and (c) and

(d) 1100°C 30% CH4 at different magnification.

Figure S3. (a) and (b) represents the SEM of 1000oC and 30%CH4 after 1st and 5th cycle (c) and (d) represents 1000oC and 50% CH4 after 1st and 5th cycle.

Table S2: Methane conversion for different temperatures at constant methane feed of 30%.

| Temp (oC) | CH4 conversion (%) |
| --- | --- |
| 900 | 20.11±2.5 |
| 950 | 21.3±0.39 |
| 1000 | 44±1.5 |
| 1050 | 46±0.44 |

Table S3: Assumption adopted in simulation of OXY-CC-CL-K

| Unit | Parameters |
| --- | --- |
| ASU | O2 purity: 99.9% (by volume); O2 and N2 delivery pressure: 1.2 bars; O2 compression pressure: 26 bars; No use of N2 was considered in the layout. |
| Turbo Expander (TURBOEXP) | Pin, NG: 70 bars; Expansion Ratio: 35; Tin, NG: 325o C. |
| Combustion Chamber | Excess Oxygen over stoichiometry for oxy-combustion: 5%; ΔPdrop: 0.1 bar; Pcombustor for COMB-1: 26 bars and COMB-2: 18 bars |
| Reduction Reactor (RED) and Combustion Chamber (COMB-1) | Reactors were modelled separately with complete heat integration. RED is modelled as moving bed reactor with COMB-1 as RGIBBS. Working pressure: 26 bars in Combustor Side and PRED: 2 bars; Non-stoichiometry generated in RED: 0.29; TRED: 1000oC; |
| Oxidation reactor (OXI) | Reactor Type: Moving bed and Adiabatic, jacketed for high-temperature steam generation Toc,inlet: 1000oC; Toc,outlet: 1350oC; Tgas, inlet: 921oC; Tgas,outlet: 1120oC; POXI: 2 bars. |
| CO2 Drying and Compression | Pdelivery: 110 bars; Tdelivery: 40oC; ηisentropic, compressor: 90%; ηmechanical,compressor: 98%. |
| Gas Turbine/ Expander | ηisentropic, turbine: 90%; Maximum pressure ratio: 18:1; Pdischarge: 1.04 bar; TIT: 1373oC |
| Steam Turbine and HRSG | Single Stage Expansion; ηisentropic, turbine: 90% IP Psteam: 150 bars; Pcondenser: 0.04 bar; ηisentropic, pump: 0.8; All of the steam generated in gasification island, chemical looping and syngas cooling unit was expanded together; Tmin, approach: 10oC, no pressure drop. |

Table S4. Stream flow rate data of the layout of OXY-CC-CL and OXY-CC-CL-K

| State | Fluid/solid | Mass Flow | Pr. | Mass Flow | Pr. |
| --- | --- | --- | --- | --- | --- |
|  |  | kg/s | bar | kg/s | bar |
| 1 | Air | 335.028 | 1.013 | 342.512 | 1.013 |
| 1a | Compressed O2 | 76.684 | 26 | 78.397 | 26 |
| 1b | Compressed N2 | 256.334 | 1.2 | 262.060 | 26 |
| 2 | Fuel | 20.486 | 70 | 20.486 | 70 |
| 2a | Fuel after Initial preheater | 20.486 | 70 | 20.486 | 70 |
| 2b | Fuel after Expander | 20.486 | 2 | 20.486 | 2 |
| 2c | Fuel into RED Reactor | 20.486 | 2 | 20.486 | 2 |
| 3 | Syngas 1-1, RED Outlet | 33.802 | 2 | 28.329 | 2 |
| 4 | Syngas 1-2 | 33.802 | 2 | 28.329 | 2 |
| 5 | Syngas 1-3 | 33.802 | 2 | 28.329 | 2 |
| 6 | Syngas 1-4, to Compressor Inlet | 33.802 | 2 | 28.329 | 2 |
| 7 | Syngas 1-5, Compressor Outlet | 33.802 | 26.1 | 28.329 | 26.1 |
| 8 | Hot GT Inlet Gas | 503.838 | 26 | 562.41 | 26 |
| 9 | GT1 Exhaust Gas | 503.838 | 2 | 562.41 | 2 |
| 10 | GT1 Second Stage Inlet Gas | 458.372 | 2 | 530.803 | 2 |
| 11 | GT1 Second Stage Exhaust Gas | 458.372 | 1.02 | 530.803 | 1.02 |
| 12 | Exhaust Gas to OXI | 45.466 | 2 | 31.609 | 2 |
| 13 | Exhaust Gas Inlet to OXI | 45.466 | 2 | 31.609 | 2 |
| 14 | Syngas 2-1, OXI Outlet | 32.15 | 2 | 23.774 | 2 |
| 15 | Syngas 2-2, Inlet to HRSG2 | 32.15 | 2 | 23.774 | 2 |
| 16 | Syngas 2-3, Exhaust from HRSG 2 | 32.15 | 2 | 23.774 | 2 |
| 17 | Syngas 2-4, Inlet to Compressor | 30.175 | 2 | 23.140 | 2 |
| 18 | Syngas 2-5, Compressor Outlet | 30.175 | 18.1 | 23.140 | 18.2 |
| 19 | Hot GT2 Inlet Gas | 166.924 | 18 | 111.957 | 18 |
| 20 | Exhaust Gas GT2 | 166.924 | 1.02 | 111.957 | 1.02 |
| 21 | Mix Exhaust Gas, to HRSG | 625.296 | 1.02 | 642.758 | 1.02 |
| 22 | Exhaust Gas HRSG Outlet | 625.296 | 1.02 | 642.758 | 1.02 |
| 23 | Exhaust Gas To Condenser | 625.296 | 1.02 | 642.758 | 1.02 |
| 24 | H2O liquid separated | 42.104 | 1.02 | 43.471 | 1.02 |
| 25 | Pure CO2 Captured | 583.192 | 1.02 | 599.289 | 1.02 |
| 26 | CO2 to Sequestration Compressor | 55.092 | 1.02 | 54.513 | 1.02 |
| 27 | CO2 Compressed for Sequestration | 55.092 | 110 | 54.513 | 110 |
| 28 | CO2 to Combustor 1 | 405.237 | 1.02 | 465.286 | 1.02 |
| 29 | CO2 to Combustor 2 | 122.863 | 1.02 | 79.218 | 1.02 |
| 30 | CO2 inlet to Combustor 1 | 405.237 | 26.1 | 465.286 | 26.1 |
| 31 | CO2 inlet to Combustor 2 | 122.863 | 18.1 | 79.218 | 18.1 |
| 32 | Oxygen Heated for Combustor 1 | 64.799 | 26 | 68.798 | 26.1 |
| 33 | Oxygen Throttled for Combustor2 | 13.885 | 18 | 9.600 | 18.1 |
| 34 | Oxygen Heated for Combustor 2 | 13.885 | 18 | 9.600 | 18.1 |
| 35 | Saturated Water after Pump | 188.973 | 150 | 183.035 | 150 |
| 36 | Sat. Water to HRSG 1 | 171.577 | 150 | 164.732 | 150 |
| 37 | Sat. Water to HRSG 2 | 17.396 | 150 | 18.304 | 150 |
| 38 | Steam from HRSG 1 | 171.577 | 150 | 164.732 | 150 |
| 39 | Steam from HRSG 2 | 17.396 | 150 | 18.304 | 150 |
| 40 | Steam Inlet to ST | 188.973 | 150 | 183.035 | 150 |
| 41 | Steam Exhaust from ST | 188.973 | 0.05 | 183.035 | 0.05 |
| 42 | Condensate | 188.973 | 0.05 | 183.035 | 0.05 |
| 43 | CeO2 out from OXI | 286.492 | 2 | 284.669 | 2 |
| 44 | CeO2 to RED | 286.492 | 2 | 284.669 | 2 |
| 45 | Ce2O3/CeO2-δ out from RED | 273.176 | 2 | 276.928 | 2 |

Table S5. Syngas composition from the Reduction Reactor (RED) and the Oxidation Reactor (OXI) of the Chemical Looping Unit (CL)

|  | Thermodynamics |  | Kinetics |  |
| --- | --- | --- | --- | --- |
| Mole Fraction (%) | From RED | From OXI | From RED | From OXI |
| CO | 28.43 | 61.4 | 22.41152 | 48.47176 |
| H2 | 54.23 | 9.84 | 44.82303 | 12.8039 |
| CO2 | 0.85 | 18.87 | 0.275885 | 34.19911 |
| H2O | 0.35 | 9.63 | 0 | 4.40159 |
| CH4 | 15.71 | Trace | 29.45483 | Trace |
| N2 | 0.43 | 0.26 | 0.55177 | 0.0959439 |
| Ethane | Trace | Trace | 2.31743 | Trace |
| Propane | Trace | Trace | 0.165531 | Trace |
| Total | 100 | 100 | 100 | 100 |